Hospital Network Design using Cisco Packet Tracer

Subtitle: Network Infrastructure for a Multi-Department Hospital

| ID | Name | Email |
| --- | --- | --- |
| Fa-2024/BSSE/015 | Haseeb Tariq | muhammadhaseebtariqt495@gmail.com |

Course: Computer Networks

Instructor: Mam Farwa Khan

Table of Contents

1. Introduction

2. Objective

3. Network Design Overview

4. Network Topology Diagram

5. IP Addressing Scheme

6.No of Components Used

7. Network Services Used

8. Routing Implementation

9. Testing and Results

10. Conclusion

1. Introduction

This project demonstrates the design, configuration, and implementation of a fully connected and service-rich network for a modern Hospital Management System using Cisco Packet Tracer. In today’s healthcare environment, fast communication, accurate patient data access, and secure information sharing are critical. To reflect this real-world requirement, the hospital network has been built with multiple interconnected departments and centralized service infrastructure.

The network includes key hospital units such as the Emergency Ward, ICU, General Ward, Pharmacy, Laboratory, and Administrative Section. Each unit is connected through routers, switches, and end-user devices forming individual local area networks (LANs), ensuring smooth workflow and minimizing congestion. A dedicated server room hosts core systems—including DNS, DHCP, HTTP, FTP, and SMTP—that support crucial functions like patient record management, web-based hospital applications, automated IP addressing, internal file sharing, and email communication.

Logical network segmentation has been used to improve performance, enhance security, and prevent unnecessary broadcast traffic between unrelated departments. Cloud connectivity is also simulated to represent access to external services, remote consultations, or online medical databases. Overall, the implemented topology demonstrates how networking technology forms the backbone of a hospital’s daily operations, ensuring fast, reliable, and safe delivery of healthcare services.

2. Objective

The objective of this project is to design and implement a secure, scalable, and professionally structured local area network (LAN) for a multi-department hospital using Cisco Packet Tracer. The network aims to provide uninterrupted communication between medical units, support critical patient data systems, and ensure efficient resource sharing within the hospital environment.

Key goals of this project include:

- Establishing a well-organized and secure LAN across various hospital departments, ensuring that emergency care, patient wards, administrative areas, and laboratory units remain connected at all times.

- Developing a logical IP addressing scheme through subnetting to separate departmental traffic, enhance network performance, and reduce broadcast congestion.

- Deploying core network services such as DHCP for automated IP allocation, DNS for hostname resolution, HTTP for web-based hospital applications, FTP for secure file transfer, and SMTP for internal email communication.

- Implementing efficient routing mechanisms, including Static Routing and RIP, to enable reliable inter-network communication while supporting future expansion or reconfiguration.

- Conducting functional and connectivity testing between all network nodes, ensuring that PCs, routers, servers, and hospital service units can exchange data seamlessly.

Through these objectives, the project demonstrates the application of networking concepts in a practical, real-world healthcare environment, highlighting how digital infrastructure supports operational efficiency in modern hospitals.

3. Network Design Overview

The hospital network is designed using a modular and hierarchical architecture to ensure reliability, scalability, and efficient data flow across all departments. Each department operates within its own logical network while remaining interconnected through routing devices and centralized services.

- Emergency Ward

  - Devices: Personal Computers (PCs), Switch, Router

  - Purpose: Supports patient admission processes, emergency case documentation, and rapid access to medical records during critical situations. The dedicated router ensures uninterrupted connectivity during high traffic conditions.

- Intensive Care Unit (ICU)

  - Devices: PCs, Network Printer, Switch

  - Purpose: Facilitates real-time access to critical patient records, monitoring data, and report printing for medical staff responsible for intensive care patients.

- General Ward

  - Devices: PCs, Router, Switch

  - Purpose: Enables nurses and healthcare staff to update patient treatment information, manage daily medical activities, and communicate efficiently with doctors and laboratory services.

- Pharmacy Department

  - Devices: PCs, Router

  - Purpose: Manages pharmaceutical inventory, prescription validation, and medicine distribution while ensuring secure access to patient medication data.

- Laboratory Department

  - Devices: PCs, Router, Server

  - Purpose: Handles laboratory test entries, result generation, and report sharing with other departments, allowing physicians to access diagnostic data promptly.

- Administration and Reception

  - Devices: PCs, Switch

  - Purpose: Responsible for patient registration, appointment scheduling, billing, and administrative record management, serving as the first point of interaction within the hospital.

- Server Room

  - Devices/Services: DNS, DHCP, HTTP, FTP, SMTP Servers

  - Purpose: Acts as the centralized service hub for the hospital, providing automated IP address allocation, name resolution, web-based hospital applications, internal file transfers, and email communication.

- Cloud / ISP

  - Devices: Router

  - Purpose: Simulates internet connectivity, enabling access to external medical resources, cloud-based services, and remote communication facilities.

4. Network Topology Diagram

5.IP Addressing Scheme

| Department / Area | Network Address | Subnet Mask | CIDR Notation | Purpose |
| --- | --- | --- | --- | --- |
| Emergency Ward | 192.168.10.0 | 255.255.255.0 | /24 | Patient admission systems and emergency medical data |
| Intensive Care Unit (ICU) | 192.168.11.0 | 255.255.255.0 | /24 | Critical patient monitoring and documentation |
| General Ward | 192.168.12.0 | 255.255.255.0 | /24 | Nursing station access and patient record updates |
| Pharmacy | 192.168.15.0 | 255.255.255.0 | /24 | Inventory management and prescription services |
| Laboratory | 192.168.14.0 | 255.255.255.0 | /24 | Diagnostic testing and laboratory reporting |
| Admin / Reception | 192.168.16.0 | 255.255.255.0 | /24 | Patient registration, billing, and administration |
| Server Room | 192.168.19.0 | 255.255.255.0 | /24 | Centralized DNS, DHCP, HTTP, FTP, and SMTP services |
| Operation Theater | 192.168.13.0 | 255.255.255.0 | /24 | Operation Details ,Patient Record/History |
| IT Department | 192.168.18.0 | 255.255.255.0 | /24 | Control all services and Departments |
| Billing Department | 192.168.17.0 | 255.255.255.0 | /24 | Manages Transactions and Bills |

6.No of Components Used:

- PCs : 70

- Servers: 13

- Routers: 6

- Cloud: 1

7. Network Services Used:

DHCP - Server Room

DNS - Server Room

HTTP - Web Server

SMTP - Mail Server

FTP - Internal file sharing

Cloud Access - ISP Router

8. Routing Implementation:

To enable effective communication between all hospital departments, both Static Routing and RIP (Routing Information Protocol) are used within the network. Static routing is configured between primary hospital routers where network paths are fixed and well-defined. This approach ensures stable connectivity, better traffic control, and reduced routing overhead for critical hospital services.

RIP dynamic routing is implemented to allow routers to automatically learn routes to external and remote networks. It simplifies route management by enabling automatic updates when network changes occur. The combined use of static and RIP routing provides a balanced routing solution, ensuring reliability, scalability, and ease of network administration.

9. Testing and Results

- Ping tests successful across departments.

- Web browser could access http://hospital.com

- DNS resolved correct IP.

- FTP uploads and downloads succeeded.

- Email services functional.

10. Conclusion

The Hospital Management Network was successfully designed, configured, and implemented in accordance with established networking principles. All hospital departments were securely interconnected through a structured network architecture while maintaining logically isolated IP address spaces to enhance performance, reliability, and security.

This implementation ensures efficient data communication, controlled traffic flow, and dependable access to centralized hospital services, demonstrating the effectiveness of the proposed network design for real-world healthcare environments.